INVERTING AND NON INVERTING AMPLIFIER

1. Inverting Amplifier

Aim:-

To design an inverting amplifier using an op-amp for the given gain of −2.

Apparatus Required:-

| S.No | Components | Specification |
| --- | --- | --- |
| 1 | Operational Amplifier | IC 741 |
| 2 | Resistors | 1 kΩ, 2 kΩ |
| 3 | Breadboard/Trainer Kit | 1 |
| 4 | CRO (Cathode Ray Oscilloscope) | 1 |
| 5 | Function Generator | 1 |
| 6 | Connecting Wires | As required |

Procedure

- Connect IC 741 on the trainer kit.

- Give dual power supply (+Vcc and −Vcc) to the IC.

- Make the circuit connections as per the circuit diagram.

- Apply sinusoidal input signal from the function generator to the inverting terminal.

- Ground the non-inverting terminal.

- Connect feedback resistor (R_f) between output and inverting terminal.

- Switch ON the power supply and CRO.

- Observe input and output waveforms on the CRO.

- Verify that the output waveform is amplified and 180° out of phase with the input.

- Tabulate the readings.

Observation Table:

| S.No | Vin (V) | Vout (V) | Practical Gain | Theoretical Gain |
| --- | --- | --- | --- | --- |
| 1 | 1V | 2V | 2 | 2 |
| 2 | 2V | 4V | 2 | 2 |
| 3 | 3V | 6V | 2 | 2 |

| S.No | Vin (V) | Vout (V) | Time Period | Frequency |
| --- | --- | --- | --- | --- |
| 1 | 0.2V | -2V | 1 ms | 1KHz |
| 2 | 0.4V | -4V | 1 ms | 1KHz |
| 3 | 0.6V | -6V | 1 ms | 1KH |

Result

The working of the inverting amplifier was studied and verified successfully.

2. Non-Inverting Amplifier

Aim

To design a non-inverting amplifier using an op-amp for the given gain of 2.

Apparatus Required

| S.No | Components | Specification |
| --- | --- | --- |
| 1 | Operational Amplifier | IC 741 |
| 2 | Resistors | 1 kΩ, 2 kΩ |
| 3 | Function Generator | 1 |
| 4 | CRO | 1 |
| 5 | Dual Power Supply | ±12 V |
| 6 | Trainer Kit | 1 |
| 7 | Connecting Wires | As required |

Procedure

- Place IC 741 on the trainer kit.

- Connect dual power supply ±12 V to the IC.

- Make connections as per the circuit diagram.

- Apply sinusoidal input signal from the function generator to the non-inverting terminal.

- Connect feedback resistor (R_f) and resistor (R_1) properly.

- Switch ON the power supply and CRO.

- Observe input and output waveforms on the CRO.

- Measure input voltage, output voltage, time period and frequency.

- Verify that the output waveform is amplified and in phase with the input waveform.

- Tabulate the readings.

Observation Table:

| S.No | Vin (V) | Vout (V) | Practical Gain | Theoretical Gain |
| --- | --- | --- | --- | --- |
| 1 | 1V | 2V | 2 | 2 |
| 2 | 2V | 3V | 2 | 2 |
| 3 | 3V | 5V | 2 | 2 |

| S.No | Vin (V) | Vout (V) | Time Period | Frequency |
| --- | --- | --- | --- | --- |
| 1 | 0.2V | 2V | 1 ms | 1KHz |
| 2 | 0.4V | 4V | 1 ms | 1KHz |
| 3 | 0.6V | 6V | 1 ms | 1KHz |

Result

The non-inverting amplifier was designed and verified successfully.